DVB-Terrestrial Receiver

SMP014HDT2

Service Manual

This service manual should be used with the User Manual together.

Please read this Service Manual and User Manual carefully before service this product.

Contents

Safety Instructions	3

Generally Guide	3

Low Potential Leakage Testing	3

High Potential Leakage Testing	3

Protect Electro-Static(ES)-sensitivity Device from Electro-Static Discharge …………………………………………. ………………………………4

Electrical Specifications	5

Mechanical Exploded View Drawing	6

Mechanical Parts List	6

Packaging and Accessories	7

Packaging Exploded View Drawing	7

Appendix 1 Flowchart and Circuit Diagram	8

Flowchart	9

Wiring Diagram	10

Circuit Diagram of Decoding Board	11-14

Appendix 2 Silkscreen of PCB	15

PCB Diagram of Decoding Board Top	16

PCB Diagram of Decoding Board Bottom	17

Appendix 3 Component List	18

Component List of Decoding Board	19-23

# Safety Instructions

## Generally Guide

- Please check electric circuits before maintenance and change the damaged or over heated components if short-circuit has been found.

- Please check all protective devices have been installed well after maintenance, like whether insulation covering and paper has been installed well or not.

- In order to avoid electric shock ,please proceed following electrical leakage tests after maintenance.

### Low Potential Leakage Testing

    - Take out power cord from an AC outlet and connect a length of wire between the two leads of the plug.

    - Use Gear R x 10K of the voltmeter to measure the resistance between shorted-out AC plug and exposed metallic parts like screw cap, control shaft etc. which shall be infinite.

Picture 1

### High Potential Leakage Testing

    - As shown in Picture 1, connect a resistor of 1.5K, 10W and capacitor of 0.15 uF between exposed metallic part and well grounded devices (water pipe etc.).

    - Plug AC power cord directly into the AC socket. Do not use insulated transformer to test.

    - Use 1000 Ohm/V or more sensitive AC voltmeter to measure AC voltage of the resistor.

    - Turn over the AC jack and plug into the socket again to iterate the inspection as above.

    - Inspect the voltage of the resistor between other exposed metallic parts and the earth in the same way.

    - Any parts’ voltage of the resistor should not over 0.75Vrms. A leakage testing machine with voltage over 2,500 V can also be used for this inspection in which case the electric leakage should not be over 0.5mA. When the leakage exceeds that limit, electric shock may occur. Please check and repair again before hand it over to users.

- Protect Electrostatic-Sensitive Devices from Electrostatic Discharge

Some solid states made of semiconductors materials can be easily damaged by commonly static charges, those components are usually called electrostatic-sensitive devices. Such like integrated circuits, laser diodes and field effect devices. The following tips will help you to reduce the impacts on those components while electrostatic discharging.

  - 1.Please release static which build-up on human body before handling electrostatic-sensitive devices by using grounded tools. The antistatic strap which can be found in the market will be a good choice.

  - 2.Please install the electrostatic-sensitive devices on conductor products such as aluminum foil to prevent static build-up after disassembling from this DVT receiver.

  - 3.The soldering iron must be earthed while soldering and unsoldering the electrostatic-sensitive devices.

  - 4.Only antistatic solder can be used for electrostatic-sensitive devices disassembly. The electrostatic-sensitive devices will be damaged by static without ESD prevention solder while disassembling.

  - 5.Do not use Freon Volatile which may damage the electrostatic-sensitive devices by discharging static.

  - 6.Do not take the new electrostatic-sensitive devices from the antistatic protection package unless you are ready for installation. (Most electrostatic-sensitive devices will be packed with anti-static foam, foil or similar conductive materials. And a lead wire to prevent short circuit.)

  - 7.Please contact the core or circuit parts of the device to be installed with ESD protection package before carry out the new electrostatic-sensitive devices from it. And make sure no power supply on the device and remember other precautions.

  - 8.Try to reduce body movements while assembling or disassembling electrostatic-sensitive devices. (Clothes made of fabrics will build-up static by attrition. Feet lifts up from floor will also build-up static.)

# Electrical Specifications

| A Audio Section （MPEG-1 Layer Ⅱ, R. L Track Output） |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # No. | ITEM |  | UNIT |  | REQUIREMENTS |  |  |  | Test Environment |  |
| 1 | Audio Output Level |  | V |  | 1.0～2.0 |  |  |  | Output impedance is 10KΩ 1KHz 0dB |  |
| 2 | Frequency Characteristics |  | dB |  | +1/-2.0 |  |  |  | 20Hz-60Hz |  |
|  |  |  |  |  | 0.5 |  |  |  | 60Hz-17.5KHz |  |
|  |  |  |  |  | +1/-3 |  |  |  | 17.5KHz-20KHz |  |
| 3 | S/N |  | dB |  | ≥70 |  |  |  | 1KHz 0dB weighting |  |
| 4 | L/R Channel Separation |  | dB |  | ≥60 |  |  |  | N-CBAR100.TS f= 1KHz P-CBAR75.TS |  |
| 5 | L/R Channel Level Difference |  | dB |  | ≤0.5 |  |  |  | 60Hz-18KHz |  |
| 6 | Audio THD |  | % |  | ≤1 |  |  |  | 60Hz-18KHz |  |
| 7 | Digital Coaxial Output Level |  | Vp-p |  | 0.5±20% |  |  |  | 75Ω Load |  |
| B Video (MPEG-2MP＠ML) |  |  |  |  |  |  |  |  |  |  |
| 1 | Output range | Video |  |  |  | Vp-p | 1.015% |  |  | 75Ω Load |
|  |  | Brightness/RGB |  |  |  |  | 0.7±10% |  |  |  |
|  |  | Sync |  |  |  |  | 0.308% |  |  |  |
| 2 | Frequency Characteristics (75Ω) |  |  |  |  | dB | 0.5 |  |  | 0.5-4.8MHz |
|  |  |  |  |  |  | dB | ≤+0.5/-1.0 |  |  | 4.8-5.0MHz |
|  |  |  |  |  |  | dB | ≤+0.5/-4 |  |  | 5.0-5.5MHz |
| 3 | Brightness S/N |  |  |  |  | dB | ≥56 |  |  | WTD 5MHz |
| 4 | Chromatic S/N |  |  |  |  | dB | AM≥58 |  |  | Load 75 |
|  |  |  |  |  |  |  | PM≥51 |  |  |  |
| 5 | Differential gain（DG） |  |  |  |  | % | ≤5 |  |  | Load 75 |
| 6 | Differential phase（DP） |  |  |  |  | ° | ≤5 |  |  | Load 75 |
| 7 | Brightness non-linear distortion |  |  |  |  | % | ≤5 |  |  | Load 75 |
| 8 | △τ chrominance-luminance delay inequality |  |  |  |  | ns | ≤30 |  |  | Load 75 |
| 9 | △K Chrominance-luminance Gain Inequality |  |  |  |  | % | ≤5 |  |  | Load 75 |
| 10 | Brightness Waveform Distortion |  |  |  |  | % | ≤3 |  |  | Load 75 |
| 11 | Chrominance Subcarrier Offset is not more than |  |  |  |  | Hz | 200 |  |  | Load 75 |
| C Demodulation |  |  |  |  |  |  |  |  |  |  |
| 1 | Input Frequency Range |  | MHz | 174~230MHz,470～862MHz (VHF/UHF) |  |  |  |  |  |  |
| 2 | Input Level Range |  | dBm | -75～-20 |  |  |  |  |  |  |
| 3 | Frequency Offset |  | MHz | -0.4MHz~+0.74MHz |  |  |  |  |  |  |
| D Power Supply (~100-240V) |  |  |  |  |  |  |  |  |  |  |
| 1 | +5V |  | mA | 1200 |  |  |  | 5% |  |  |
| 2 | +5V Active Antenna Amplifier |  | mA | 30 |  |  |  | 5V overload protection |  |  |
| E Others |  |  |  |  |  |  |  |  |  |  |
| 1 | Free Fall |  | Meets QJ/ET08.02-2005 Standards |  |  |  |  |  |  |  |
| 2 | Remote Control Distance |  | M | ≥8 |  |  |  | In line |  |  |
|  |  |  |  | ≥6 |  |  |  | With range of 30° |  |  |
| 3 | Rated Power Consumption |  | W | 8 |  |  |  |  |  |  |

# Mechanical Exploded View Drawing

## Mechanical Parts List

| NO. | Part No. | Part Name | Qty | Notes |
| --- | --- | --- | --- | --- |
| 1 | 4110-2152-000H | Top Cover | 1 | ABS |
| 2 | 2104-2102-A00H | Decoding Board | 1 |  |
| 3 | 4111-2152-000H | Bottom Cover | 1 | ABS |
| 4 | 3000-4000-0800 | Feet Pad | 2 | Rubber, Black |
| 5 | 3211-2008-0005 | Screws ST2x8PANi | 2 |  |
| 6 |  |  |  |  |

Packaging and Accessories

## Packaging Exploded View Drawing

Material List

| Appendix 1 Flowchart and Circuit Diagram NO. | Material No. | Name | Qty |
| --- | --- | --- | --- |
| 1 | 4401-2152-000H | White Box | 1 |
| 2 | ST2.025.0702MXR1 | User Manual | 1 |
| 3 | 2301-1719-000H | Remote Control | 1 |
| 4 | 4413-1812-1100 | Bubble Bag 180*210mm | 1 |
| 5 | 2102-0105-100B | Adaptor | 1 |

## Flowchart

## Wiring Diagram

## Circuit Diagram of Decoding Board

Flowchart

Wiring Diagram

Decoding Board Schematic Diagram (1)

Decoding Board Schematic Diagram (2)

Decoding Board Schematic Diagram (3)

Decoding Board Schematic Diagram (4)

# Appendix 2 Silkscreen of PCB

## PCB Diagram of Decoding Board Top

## PCB Diagram of Control Board Bottom

PCB Diagram of Decoding Board Top

PCB Diagram of Decoding Board Bottom

# Appendix 3 Component List

## Component List of Decoding Board

Component List

| Component List of Decoding Board |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Material No. | Name | Specification | Position | Qty |
| 1 | 4208-2102-000H | decoding deck, printing plate | ST7.820.1525,90×70×1.2mm |  | 1 |
| 2 |  |  | Double-sided board, ROHS |  |  |
| 3 | 3260-1000-2005 | Heat Sink | ST7.061.0098,20×20×10mm, | On the top of U1 | 1 |
| 4 |  |  | Aluminum,ROHS |  |  |
| 5 | 4303-1599-000H | Shield Cover | ST6.430.0024,0.3mm | J6 | 1 |
| 6 |  |  | Tinplate,ROHS |  |  |
| 7 | 4303-0762-001H | Shield Cover | ST6.430.0016,0.3mm Tinplate | J6 | 1 |
| 8 |  |  |  |  |  |
| 9 | 3020-1000-0004 | SMD Resistor | 0Ω±5%,1/16w,0402,RoHS | R35,R57, L419 | 3 |
| 10 | 3020-1051-0002 | SMD Resistor | 5.1Ω±5%,1/16w,0402,ROHS | R214,R215 | 2 |
| 11 | 3020-1100-0008 | SMD Resistor | 10Ω±5%,1/16w,0402,RoHS | R204,R205,R206,R207,R208, | 8 |
| 12 |  |  |  | R209,R210,R211 |  |
| 13 | 3020-1220-0005 | SMD Resistor | 22Ω±5%,1/16w,0402,RoHS | R273 | 1 |
| 14 | 3020-1301-0004 | SMD Resistor | 300Ω±5%,1/16w,0402,ROHS | R307,R308 | 2 |
| 15 | 3020-1470-0002 | SMD Resistor | 47Ω±5% 1/16W 0402 | R63 | 1 |
| 16 | 3020-6750-0000 | SMD Resistor | 75Ω±1%,1/16W,0402,RoHS | R302 | 1 |
| 17 | 3020-1101-0011 | SMD Resistor | 100Ω±5%,1/16w,0402 | RT14,R251,R252 | 9 |
| 18 |  |  |  | R279,R280,R61,R97,R218, |  |
| 19 |  |  |  | R227 |  |
| 20 | 3020-1221-0008 | SMD Resistor | 220Ω±5%,1/16w,0402,RoHS | R106,R112 | 2 |
| 21 | 3020-1471-0003 | SMD Resistor | 470Ω±5%,1/16w,0402 | R107 | 1 |
| 22 | 3020-1102-0005 | SMD Resistor | 1K±5%,1/16W,0402 | R103,R246,R270 | 3 |
| 23 | 3020-1132-0002 | SMD Resistor | 1.3KΩ±5%,1/16w,0402,RoHS | R267 | 1 |
| 24 | 3020-1182-0003 | SMD Resistor | 1.8KΩ±5%,1/16w,0402,RoHS | R14,R200,R201 | 3 |
| 25 | 3020-1202-0005 | SMD Resistor | 2K±5%,1/16w,0402 | R108 | 1 |
| 26 | 3020-1472-0003 | SMD Resistor | 4.7K±5%,1/16w,0402 | R69,R74,R83,R249,R250,R259 | 6 |
| 27 | 3020-1103-0005 | SMD Resistor | 10K±5%,1/16w,0402 | R34,R51,R52,R62,R203,R255 | 9 |
| 28 |  |  |  | R268,R271,R115 |  |
| 29 |  |  |  |  |  |
| 30 | 3020-1113-0002 | SMD Resistor | 11KΩ±5%,1/16w,0402, | R272 | 1 |
| 31 | 3020-1153-0003 | SMD Resistor | 15KΩ±5%,1/16W,0402, | R269 | 1 |
| 32 | 3020-6220-0300 | SMD Resistor | 22KΩ±1%,1/16W,0402, | R26 | 1 |
| 33 | 3020-1223-0002 | SMD Resistor | 22KΩ±5%,1/16W,0402, | R274 | 1 |
| 34 | 3020-6910-0301 | SMD Resistor | 91KΩ±1%,1/16w,0402 | R23 | 1 |
| 35 | 3020-6100-0400 | SMD Resistor | 100KΩ±1%,1/16W,0402 | R25,R27 | 2 |
| 36 | 3020-1104-0006 | SMD Resistor | 100K±5%,1/16w,0402 | R22 | 1 |
| 37 | 3020-1105-0007 | SMD Resistor | 1MΩ±5%,1/16w,0402, | R60 | 1 |
| 38 | 3020-1015-0002 | SMD Resistor | 1.5Ω±5%,1/8w,0805, | R266 | 1 |
| 39 | 3020-1000-0000 | SMD Resistor | 0Ω±5%,1/8w,0805,RoHS | R1 | 1 |
| 40 | 3040-1015-0000 | chip capacitor | 1.5PF±0.25PF,50V,0402,RoHS | C168 | 1 |
| 41 | 3040-1100-0007 | chip capacitor | 10PF±5%,16V,0402,RoHS | C91 | 1 |
| 42 | 3040-1120-0000 | chip capacitor | 12PF±5%,50V,0402,RoHS | C169,C170 | 2 |
| 43 | 3040-1220-0005 | chip capacitor | 22PF±5%,16V,0402 | C25,C27 | 2 |
| 44 | 3040-1330-0003 | chip capacitor | 33PF±5%,16V,0402 | C89,C90,C174 | 3 |

Component List

| Component List of Decoding Board |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Material No. | Name | Specification | Position | Qty |
| 1 | 3040-1151-0001 | chip capacitor | 150PF±5%,50V,0402,RoHS | CT15,CT16, C167 | 3 |
| 2 | 3040-1331-0006 | chip capacitor | 330pF±10%,50V,0402 | C87 | 1 |
| 3 | 3040-1471-0004 | chip capacitor | 470PF±5%,50V,0402,RoHS | C76 | 1 |
| 4 | 3040-1682-0002 | chip capacitor | 6800PF±10%,25V,0402,RoHS | C86 | 1 |
| 5 | 3040-1223-0006 | chip capacitor | 0.022uF±10%,16V,0402 | C45,C70 | 2 |
| 6 | 3040-1103-0026 | chip capacitor | 10nF±10%,16V,0402,X5R,ROHS | CT28,C155 | 2 |
| 7 | 3040-1104-0018 | chip capacitor | 0.1uF+80/-20% 25V 0402 | C14,C19,C21,C24,C30,C32 | 58 |
| 8 |  |  |  | C36,C37,C38,C39,C41,C42 |  |
| 9 |  |  |  | C43,C44,C46,C48,C49,C50, |  |
| 10 |  |  |  | C51,C53,C54,C55,C57,C59 |  |
| 11 |  |  |  | C60,C63,C66,C68,C73,C75, |  |
| 12 |  |  |  | C77,C78,C93,C146,C147,C148, |  |
| 13 |  |  |  | C159,C163,C164,C173,C180, |  |
| 14 |  |  |  | C181,C182,C183,C191,C192 |  |
| 15 |  |  |  | C193,C194,C61C80,C83,C84 |  |
| 16 |  |  |  | C152,C157,C171C172,C72,C82 |  |
| 17 |  |  |  |  |  |
| 18 | 3040-1105-0014 | chip capacitor | 1uF+80-20%,16V,0402 | C69,C71,C156 | 3 |
| 19 | 3040-1105-0001 | chip capacitor | 1uF±20%,16V,0603,RoHS | C16 | 1 |
| 20 | 3040-1106-0007 | chip capacitor | 10uF+80-20%,16V,0805 | C65,C67,C18,C40,C47, | 10 |
| 21 |  |  |  | C154,C162,C201,C20,C23 |  |
| 22 | 3040-2227-0033 | electrolytic capacitor | 220uF±20%,6.3V,Φ6.3×5mm, | EC7 | 1 |
| 23 | 3040-2227-0038 | electrolytic capacitor | 220UF±20%,10V,Φ5×11mm,105°,RoHS | EC2,EC8 | 2 |
| 24 | 3040-2107-0001 | electrolytic capacitor | 100UF±20%,10V,Φ5×11mm,RoH | EC19 | 1 |
| 25 | 3170-3006-0336 | Chip Bead | 0603,6Ω,100mHz,DCR=0.05Ω, | FB2,FB4,FB6,FB17 | 4 |
| 26 |  |  |  |  |  |
| 27 | 3170-3004-0211 | Chip Bead | 0402,30Ω,100MHz,300mA,ROHS | FB11,FB13,FB16,FB18,FB27, | 8 |
| 28 |  |  |  | FB28,FB29,FB32 |  |
| 29 | 3170-3006-0307 | Chip Bead | 0603,120Ω,100MHz,RoHS | FB33,FB14 | 2 |
| 30 | 3170-3008-0503 | Chip Bead | 0805,120Ω,100MHz,RoHS | L29 | 1 |
| 31 | 3170-3004-0203 | Chip Bead | 1KΩ,100MHz,0402,150mA,RoHS | L38,L39 | 2 |
| 32 | 3050-1302-2003 | high-frequency chip inductor | 2.2nH±5%,0402, | L416 | 1 |
| 33 |  |  |  |  |  |
| 34 | 3050-1368-0000 | chip inductor | 68nH±5%,0402, | L32 | 1 |
| 35 |  |  |  |  |  |
| 36 | 3050-1315-1007 | White high-frequency ceramic inductors | 150nH±5% 0603 | L31 | 1 |
| 37 | 3050-1318-1003 | chip inductor | 180nH±5%,0402,ROHS | L40 | 1 |
| 38 | 3050-1327-1009 | White high-frequency ceramic inductors | 270nH±10%,0402, | L417 | 1 |
| 39 | 3050-1033-0014 | SMT Power Inductors | 3.3UH±20%,SMD43,2.5A,RoHS | L3 | 1 |
| 40 | 3050-1047-0028 | SMT Power Inductors | 4.7uH±20% SD43-4R7MC 1.7A | L14 | 1 |
| 41 |  |  |  |  |  |
| 42 |  |  |  |  |  |
| 43 |  |  |  |  |  |

Component List

| Component List of Decoding Board |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Material No. | Name | Specification | Position | Qty |
| 1 | 3060-1990-0000 | Chip Diode | BAV99,SOT-23 | D403 | 1 |
| 2 | 3060-3392-0000 | luminous diode | Φ3, astigmatism highlight, positive ,red and green | LED5 | 1 |
| 3 | 3060-5239-0400 | SMD TRANSISTOR | 2N3904 SOT23 | Q8 Q25 | 2 |
| 4 | 3060-5239-0600 | SMD TRANSISTOR | 2N3906 SOT23 | Q24,Q26 | 2 |
| 5 | 3070-1243-8000 | Receiver module | HL-2438M,38KHz,ROHS | IR1 | 1 |
| 6 | 3070-1810-0002 | SMD IC | SGM810 SOT-23 | Q30 | 1 |
| 7 | 3070-1253-2003 | SMD IC | GD25Q32BSIG 4Mbytes SO8NBW | U7 | 1 |
| 8 | 3070-1241-0001 | SMD IC | APS2410ES5-ADJ SOT23-6 | U3 | 1 |
| 9 | 3070-1242-0500 | SMD IC | APS2420ES5-ADJ SOT23-6 | U4 | 1 |
| 10 | 3070-1836-0000 | SMD IC | R836,QFN-24 | U8 | 1 |
| 11 | 3070-1701-0001 | SMD IC | MSD7T01-Z00-DA0(without dolby) | U6 | 1 |
| 12 |  |  |  |  |  |
| 13 | 3070-1111-7106 | SMD IC | 1117-S18,1.8V,SOT-223,ROHS | U9 | 1 |
| 14 | 3162-0011-0000 | HDMIsocket | GH-FBL-011N, ironcladnickeling, electroplate 1U",ROHS | J3 | 1 |
| 15 | 3160-9384-5900 | RAC Socket | AV3-8.4-59, Yellow, white and red, without earth tag ,without screw hole,ROHS | JK3 | 1 |
| 16 | 3160-5000-0000 | USB socket | ZX-USBSOCKET-A, white ROHS | J4 | 1 |
| 17 | 3110-2240-0603 | oscillator | 24MHz,20PPM, fundamental wave 49S | Y1 | 1 |
| 18 | 3110-2160-0600 | crystal oscillator | 16MHz, fundamental wave 49S,20PPM,RoHS | Y1 | 1 |
| 19 |  |  |  |  |  |
| 20 | 3160-1005-0000 | DC socket | DC JACK，DC-005 | PCN1 | 1 |
| 21 | 3200-3010-0000 | varnishedtube | Φ1.0 |  |  |
| 22 |  | varnishedtube | Φ1.0×5mm×2 | LED5 | 10 |
| 23 |  |  |  |  |  |
| 24 |  |  |  |  |  |
| 25 |  |  |  |  |  |
| 26 |  |  |  |  |  |
| 27 |  |  |  |  |  |
| 28 |  |  |  |  |  |
| 29 |  |  |  |  |  |
| 30 |  |  |  |  |  |
| 31 |  |  |  |  |  |
| 32 |  |  |  |  |  |
| 33 |  |  |  |  |  |
| 34 |  |  |  |  |  |
| 35 |  |  |  |  |  |
| 36 |  |  |  |  |  |
| 37 |  |  |  |  |  |
| 38 |  |  |  |  |  |
| 39 |  |  |  |  |  |
| 40 |  |  |  |  |  |
| 41 |  |  |  |  |  |
| 42 |  |  |  |  |  |